Bài tập chương 2

1.5 [4] Consider three different processors P1, P2, and P3 executing the same instruction set. P1 has a 3 GHz clock rate and a CPI of 1.5. P2 has a 2.5 GHz clock rate and a CPI of 1.0. P3 has a 4.0 GHz clock rate and has a CPI of 2.2.

a. Which processor has the highest performance expressed in instructions per second?

IPS (Number of Instruction per second) = fCPI

For P1:

IPS1​ =   3 GHz1.5​ = 3 x 1091.5 = 2 × 109  instructions/second

For P2:

IPS2​ =   2.5 GHz1​ = 2.5  x 1091 = 25 × 108  instructions/second

For P3:

IPS2​ =   4 GHz2.2​ = 4 x 1092.2 ≈ 1.818 × 109  instructions/second

P2 has the highest performance with 25 ×108  instructions per second

b. If the processors each execute a program in 10 seconds, find the number of cycles and the number of instructions.

ET = NC/f

NC = NI x CPI  NI = NC/CPI

For P1:

NC1  = ET x f1 = 10 x 3 GHz  = 30 x 109 cycles

NI1 = NC1CPI1 = 30 x 109 1.5 = 20 x 109  instructions

For P2:

NC2  = ET x f2 = 10 x 2.5 GHz  = 25 x 109 cycles

NI2 = NC2CPI2 = 25 x 109 1 = 25 x 109  instructions

For P3:

NC3  = ET x f3 = 10 x 4 GHz  = 40 x 109 cycles

NI3 = NC3CPI3 = 40 x 109 2.2 ≈ 18.18 x 109  instructions

c. We are trying to reduce the execution time by 30% but this leads to an increase of 20% in the CPI. What clock rate should we have to get this time reduction?

fnewfold = EToldETnew × CPInewCPIold = ETold0.7ETold × 1.2CPIoldCPIold = 127

→ fnew=1,71× fold

1.6 [20] Consider two different implementations of the same instruction set architecture. The instructions can be divided into four classes according to their CPI (class A, B, C, and D). P1 with a clock rate of 2.5 GHz and CPIs of 1, 2, 3, and 3, and P2 with a clock rate of 3 GHz and CPIs of 2, 2, 2, and 2. Given a program with a dynamic instruction count of 1.0E6 instructions are divided into classes as follows: 10% class A, 20% class B, 50% class C, and 20% class D, which implementation is faster?

|  | CPI |  |  |  |
| --- | --- | --- | --- | --- |
| P1 (2.5 GHz) | 1 | 2 | 3 | 3 |
| P2 (3 GHz) | 2 | 2 | 2 | 2 |
| Instruction | 1 x 105 | 2 x 105 | 5 x 105 | 2 x 105 |

a. What is the global CPI for each implementation?

CPIglobal(1​)= (0.1×1) + (0.2×2) + (0.5×3) + (0.2×3) = 2.6

CPIglobal(2)= (0.1×2) + (0.2×2) + (0.5×2) + (0.2×2) = 2

b. Find the clock cycles required in both cases.

NC1 =  CPIglobal1 x NItotal = 2.6 x 106 clock cycles

NC2 =  CPIglobal2 x NItotal  = 2 x 106 clock cycles

1.7 [15] Compilers can have a profound impact on the performance of an application. Assume that for a program, compiler A results in a dynamic instruction count of 1.0E9 and has an execution time of 1.1 s, while compiler B results in a dynamic instruction count of 1.2E9 and an execution time of 1.5 s.

a. Find the average CPI for each program given that the processor has a clock cycle time of 1 ns.

Ta có: ET = NI x CPI x T  →CPI= ETNI x T

CPIA = 1.1109 ×10-9 = 1.1 clock cycle per instruction

CPIB = 1.51.2 × 109 ×10-9 = 1.25 clock cycle per instruction

b. Assume the compiled programs run on two different processors. If the execution times on the two processors are the same, how much faster is the clock of the processor running compiler A’s code versus the clock of the processor running compiler B’s code?

ET = constant

ET = NI x CPI / f

f ~ NI, CPI

fAfB= NIA × CPIANIB × CPIB = 109 × 1.11.2 × 109 × 1.25 = 1.11.5= 0.733

B’s code is approximately 1.51.1 times faster than the one running compiler A’s code.

c. A new compiler is developed that uses only 6.0E8 instructions and has an average CPI of 1.1. What is the speedup of using this new compiler versus using compiler A or B on the original processor?

ETnew  = 6 x 108 x 1.1 x 10-9  = 0.66s

ETAETnew = 1.10.66 ≈1.67    The new Complier is 1.67 times faster than compiler A

ETBETnew = 1.50.66≈ 2.27	The new Complier is 2.27 times faster than compiler B