GIẢI BÀI TẬP CHƯƠNG 2

Câu 2:

lw $t1, 20($t0)

add $s0, $s1, $t1

Biểu diễn mã máy tương ứng:

1000 1101 0000 1001 0000 0000 0001 0100		# lw $t1, 20($t0)

0x8D090014

0000 0010 0010 1001 1000 0000 0010 0000		# add $s0, $s1, $t1

0x02298020

Câu 3:

Phân biệt:

MIPS sử dụng chiến lược big end

Câu 4:

blt $s0, 0, if

add $s1, $zero, $s0

j exit

if:

addi $s1, $zero, 0

exit:

li $v0, 10

syscall

Câu 6:

proc_exa:

add $t1, $s0, $s1

add $t2, $s2, $s3

sub $s6, $t1, $t2

Câu 7:

.data

greeting: .asciiz "Hello World"

.text

main:

li $v0,4

la $a0, greeting

syscall

li $v0, 10

syscall

Câu 8:

.data

name: .space 50

greeting: .ascii "Hello "

.text

li $v0, 8

la $a0, name	#Nhap vao ten mot nguoi

li $a1, 50

syscall

li $v0, 4

la $a0, greeting

syscall

li $v0, 4

la $a0, name

syscall

li $v0, 10

syscall

Câu 9:

.data

chuoi1: .asciiz "Nhap chieu dai:"

chuoi2: .asciiz "Nhap chieu rong:"

chuoi3: .asciiz "Chu vi hinh chu nhat la:"

chuoi4: .asciiz "Dien tich hinh chu nhat la:"

chuoi5: .asciiz "\n"

so2: .float 2.0

.text

li $v0, 4

la $a0, chuoi1		#In ra man hinh dong chu "Nhap chieu dai:"

syscall

li $v0, 6

syscall			#Nhap vao chieu dai cua hcn

mov.s $f1, $f0

li $v0, 4

la $a0, chuoi2		#In ra man hinh dong chu "Nhap chieu rong:"

syscall

li $v0, 6

syscall			#Nhap vao chieu rong cua hcn

mov.s $f2, $f0

li $v0, 4

la $a0, chuoi3		#In ra man hinh dong chu "Chu vi hinh chu nhat la:"

syscall

add.s $f3, $f1, $f2

l.s $f4, so2		# Tinh chu vi hcn

mul.s $f5, $f3, $f4

li $v0, 2

mov.s $f12, $f5		# Chuyen gia tri $f5 vao $f12

syscall

li $v0, 4

la $a0, chuoi5	#In ra man hinh dau xuong dong

syscall

li $v0, 4

la $a0, chuoi4	#In ra man hinh dong chu "Dien tich hinh chu nhat la:"

syscall

mul.s $f6, $f1, $f2	# Tinh dien tich hcn

li $v0, 2

mov.s $f12, $f6		# Chuyen gia tri $f6 vao $f12

syscall

li $v0, 10

syscall

Câu 10:

.data

prompt1: .asciiz "Nhap n: "

prompt2: .asciiz "Nhap k: "

output: .asciiz "Ket qua cua to hop la: "

.text

# Input n

li $v0, 4

la $a0, prompt1          # Print "Nhap n:"

syscall

li $v0, 5                # Syscall code for reading an integer

syscall

add $t0, $v0, $zero            # Store n in $t0

add $a0, $t0, $zero            # Move n to $a0 for factorial

jal fact                 # Call factorial function

add $t1, $v0, $zero            # Store n! result in $t1

# Input k

li $v0, 4

la $a0, prompt2          # Print "Nhap k:"

syscall

li $v0, 5                # Syscall code for reading an integer

syscall

add $t2, $v0, $zero             # Store k in $t2

# Calculate k!

add $a0, $t2, $zero            # Move k to $a0 for factorial

jal fact                 # Call factorial function

add $t3, $v0, $zero            # Store k! result in $t3

# Calculate (n - k)!

sub $t4, $t0, $t2        # Calculate n - k

add $a0, $t4, $zero            # Move (n - k) to $a0 for factorial

jal fact                 # Call factorial function

add $t5, $v0, $zero            # Store (n - k)! result in $t5

# Calculate k! * (n - k)!

mul $t6, $t3, $t5        # k! * (n - k)!

# Calculate n! / (k! * (n - k)!)

div $t1, $t6             # n! / (k! * (n - k)!)

mflo $t7                 # Move result to $t7

# Output the result

li $v0, 4

la $a0, output           # Print "Ket qua cua to hop la:"

syscall

li $v0, 1                # Syscall code for printing an integer

add $a0, $t7, $zero            # Move result to $a0

syscall

li $v0, 10               # Exit

syscall

# Function to calculate factorial

fact:

# Calculate factorial of $a0 (n)

li $v1, 1                # Initialize $v1 to 1 (factorial result)

factorial_loop:

beq $a0,$zero, factorial_done # If $a0 (n) is zero, exit loop

mul $v1, $v1, $a0        # Multiply $v1 by $a0

subi $a0, $a0, 1         # Decrement $a0

j factorial_loop         # Repeat loop

factorial_done:

add $v0, $v1, $zero            # Move result to $v0

jr $ra                   # Return

Câu 11:

.data

int_array: .word 7,10,4,25,20,1,18,3,16,8

len: .word 10

output: .asciiz "Max value:"

.text

main:

li $v0, 4 		#Print "Max value:"

la $a0, output

syscall

la $s0,int_array

lw $t1, len

lw $a0, 0($s0)

loop:

addi $t1, $t1, -1		# len=len-1

beq $t1, $zero, exit 		# len==0 thi tra ve exit

addi $s0, $s0, 4		# &a[i] +4

lw $t0, 0($s0)			# $t0 =a[i]

bge $a0,$t0, non_max		# if( $a0 >= a[i]) nhay den non_max

addi $a0, $t0, 0		# dieu kien sai thi a[i] == max

non_max:

j loop

exit:

li $v0, 1	# In ra gia tri trong thanh ghi $a0

syscall

li $v0, 10

syscall

Câu 12:

.data

float_array: .float 7,10,4,25,20,1,18,3,16,8

len: .word 10

output: .asciiz "Average:"

.text

main:

li $v0, 4 		#Print "Average:"

la $a0, output

syscall

la $s0,float_array

lw $t1, len

addi $sp, $sp, -4	# Lay gia tri len dua vao ngan xep

sw $t1, 0($sp)

loop:

l.s $f0, 0($s0)			# $t0 =a[i]

addi $s0, $s0, 4		# &a[i] +4

add.s $f2, $f2, $f0		# $t2 = $t2 + a[i]

addi $t1, $t1, -1		# len=len-1

beq $t1, $zero, exit 		# len==0 thi tra ve exit

j loop

exit:

lw $t1, 0($sp)		# Dua gia tri tu ngan xep vao lai #t1

addi $sp, $sp, 4

li $v0, 2	# In ra gia tri trong thanh ghi $a0

mtc1 $t1, $f1

cvt.s.w $f1, $f1	#Chuyen so nguyen thanh so thuc

div.s $f12, $f2, $f1

syscall

li $v0, 10

syscall

Câu 13: